OMEN Kilo / issue 1

Technical Documentation

INTRODUCTION

===========

The OMEN Kilo computer kit is a low-cost computer trainer, based on the MC6809 / HD6309 CPU. It has these features:

- MC6809 CPU working at 1.8432 MHz

- 32 kB RAM

- 8 kB EEPROM

- Serial port up to 115.200 Bd / MC6850 ACIA

- Application system bus

ASSEMBLY INSTRUCTIONS

===================

- Solder sockets for the integrated circuits

- Test all soldered connections

  - Test if all pins are well connected

  - Check if GND is not short connected to Vcc

  - Check if each IC has properly connected GND and Vcc

- Solder all passive parts /capacitors, diode, resistors, push button, crystals/

- Connect the power adapter and check

- Insert the CPU into its socket /keep the proper orientation!/ and try to power it up. Check if oscillator lives /at CPU pin 34/

- Insert the other ICs. Again: keep the proper orientation! Bad orientation can damage the IC!

- Connect the serial pins TxD, RxD and GND /pinhead SERIAL/ to the TTL-to-USB converter

- Start the serial terminal on your PC, select proper serial port and set the parameters to 115.200 Bd, 8 data bits, no parity, 1 stop bit.

- Power your Kilo and check the terminal.

MONITOR

=======

The main software for OMEN Kilo issue 2 is the Kilo Monitor version 2.0. Source codes are available on GitHub.

Basic commands are:

Daddr – Memory dump. Next line with ENTER, SPACE aborts dumping, BACKSPACE displays previous addresses.

Maddr – Modify memory byte by byte. Use ENTER, SPACE and BACKSPACE in the same way as in DUMP.

Gaddr – Go to program (JMP)

W from to – export memory as SREC

X from to – export memory as HEX

: - start HEX file to load. You can just send HEX file directly to the terminal to load data or program into the memory.

U – start a use module. Kilo EEPROM contains one preprogrammed module, the TINY BASIC interpreter. See source code to further information about the modular concept.

THE BOARD

========

Jumpers and pin headers

--------------------

SERIAL: Serial port. Pins are RxD, TxD, GND from up to the bottom, RxD is near the edge.

WREN: PLEASE DO NOT USE, just solder the middle pin and the left pin /close to the BANK switch/ together. I am sorry.

BANK: EEPROM bank selector – only for AT28C256. Leave it open.

NMIX: NMI enabler. Close to disable non-maskable interrupts, open to leave it on the application bus. Connect the 10k resistor to make it “open-collector signal”.

IRQX: Interrupt enabler. Close to disable maskable interrupts, open to leave it on the application bus. Connect the 10k resistor to make it “open-collector signal”.

FIRQX: Fast Interrupt enabler. Close to disable maskable interrupts, open to leave it on the application bus. Connect the 10k resistor to make it “open-collector signal”.

WAITX, DMAX: Connect the 10k resistor or short wire, if you does not plan using these features.

SYSTEM APPLICATION CONNECTOR

=========================

This connector is on the right edge of board. Pin 1 is on the upper side, next to the SYSTEM label.

Pins:

- /WR

- D0

- D1

- D2

- D3

- D4

- D5

- D6

- D7

- A0

- A1

- A2

- /RD

- IO1    --- 9000h – 93FFh

- IO2    --- 8800h – 8BFFh

- IO3    --- 9800h – 9BFFh

- IO4    --- 8400h – 87FFh                             IOx signals are decoded by 74138

- IO5    --- 9400h – 97FFh

- IO6    --- 8C00h – 8FFFh

- IO7    --- 9C00h – 9FFFh

- Vcc

- GND

- IRQ   --- See the IRQX jumper

- PHI2

- /RESET

- /WAIT --- See the WAITX jumper

- /BUSRQ --- See the DMAX jumper

- /BUSACK

- /FIRQ --- See the FIRQX jumper

PERIPHERAL IC ADDRESSES
=====================

ACIA 68B50:
--------

83FEh – Command Register

83FFh - Data Register

MEMORY MAP
==========

0000h – 7FFFh  - System RAM 32k

8000h – BFFFh – I/O space

C000h – DFFFh – EEPROM 8k shadow

E000h – FFFFh – EEPROM 8k